LAB MANUAL

Name:  Tuba Jan

Cms-Id:  Csc-25f-602

Course: Digital Logic Design Course Supervisor:  Sir Amasil

Instructor:  Sir Sunil Kumar

Course Code:  CSC-108

Semester: Second

Program: BSCS

Section:  IIC (E)

# CERTIFICATE

### Department of Computer Science

This is to certify that Ms. 		So/Do 	 having

Roll No. 	 Has successfully completed laboratory work during Semester Second 2026.

Lab Instructor 	Course Supervisor

TABLE OF CONTENTS

| S.NO | INDEX |
| --- | --- |
| # | Introduction |
| 1 | Logic Gates (AND, OR, NOT) |
| 2 | Universal Gates (NAND, NOR) |
| 3 | XOR and XNOR Gates |
| 4 | Half Adder and Full Adder |
| 5 | Half Subtractor and Full Subtractor |
| 6 | Multiplexers (2:1, 4:1 MUX |
| 7 | DEMULTIPLEXERS (1:2 AND 1:4) |
| 8 | ENCODERS |
| 9 | DECODERS |

LAB 1: INTRODUCTION TO LOGIC GATES (AND, OR, NOT)

Objective:

▪ Understand the basics of digital logic.

▪ Learn to implement logic gates in Logisim.

▪ Verify truth tables of NOT, AND, and OR gates.

▪ Observe how binary inputs control outputs.

Components Used:

▪ Logic Gates: AND, OR, NOT

▪ Input Pins

▪ Output LEDs

▪ Wires

▪ Theory and Explanation:

▪ Digital circuits use binary signals: 0 = LOW, 1 = HIGH.

▪ AND Gate: Outputs HIGH only when all inputs are HIGH. Logic Expression: Y = A·B

▪

TRUTH TABLE:

A | B | Y

0 | 0 | 0

- | 1 | 0

- | 0 | 0

1 | 1 | 1

OR GATE: Outputs HIGH if at least one input is HIGH.

Logic Expression: Y = A + B

Truth Table:

A | B | Y

0 | 0 | 0

- | 1 | 1

- | 0 | 1

1 | 1 | 1

NOT GATE: Outputs the inverse of input.

Logic Expression: Y = ¬A Truth Table: A | Y

- | 1

- | 0

These gates are building blocks for all digital circuits.

LAB TASKS:

Open Logisim and create a new project.

Place AND, OR, NOT gates.

Connect input pins and LED outputs.

Test all input combinations.

Observe LED outputs.

Theory Questions:

  - Explain how AND and OR gates differ.

  - What is the purpose of a NOT gate?

Practical Questions:

  - Test AND gate with all input combinations. When does LED glow?

For AND gate, apply all four possible binary combinations to the inputs A and B and observe the output Y.

| Input A | Input B | Output Y (LED Status) |
| --- | --- | --- |
| 0 (LOW) | 0 (LOW) | 0 (OFF) |
| 0 (LOW) | 1 (HIGH) | 0 (OFF) |
| 1 (HIGH) | 0 (LOW) | 0 (OFF) |
| 1 (HIGH) | 1 (HIGH) | 1 (ON/GLOW) |

The LED glows (outputs a HIGH signal) only when all inputs are HIGH (1).

  - Test OR gate with all input combinations.

In an OR gate, the circuit is designed to produce a HIGH output if at least one of its inputs is HIGH.

| Input A | Input B | Output Y (LED Status) |
| --- | --- | --- |
| 0 (LOW) | 0 (LOW) | 0 (OFF): The LED remains off because no input is HIGH. |
| 0 (LOW) | 1 (HIGH) | 1 (ON): The LED glows because input B is HIGH. |
| 1 (HIGH) | 0 (LOW) | 1 (ON): The LED glows because input A is HIGH. |
| 1 (HIGH) | 1 (HIGH) | 1 (ON): The LED glows because both inputs are HIGH. |

  - Test NOT gate behavior

The NOT gate, also known as an inverter, the purpose is to produce the inverse of the input.

| Input (A) | Output Y (LED Status) |
| --- | --- |
| 0 (LOW) | 1 (ON): The LED glows because the gate inverts the LOW signal to HIGH. |
| 1 (HIGH) | 0 (OFF): The LED remains off because the gate inverts the HIGH signal to LOW. |

LAB 2: UNIVERSAL GATES (NAND AND NOR)

Objective:

Components Used:

    - NAND, NOR gates

    - Input Pins

    - Output LEDs

    - Wires Theory:

1. NAND Gate: Output HIGH except when all inputs are HIGH. Logic: Y = ¬(A·B)

A NAND gate is a combinational logic gate that produces an output HIGH (1) in all cases except when all inputs are HIGH (1).

2. NOR Gate: Output HIGH only when all inputs are LOW. Logic: Y = ¬(A + B)

A NOR gate is a combinational logic gate that produces an output HIGH (1) only when all inputs are LOW (0).

3. Universal gates can implement any logic function.

NAND and NOR are universal gates.A NOT gate can be made using a single NAND or NOR gate

LAB TASKS:

- Implement NAND and NOR in Logisim.

- Use NAND/NOR to build NOT, AND, OR gates.

- Test all input combinations.

- Verify truth tables.

Theory Questions:

- How can NAND implement NOT gate?

- Why are NAND and NOR called universal gates?

Practical Questions:

- Build AND using NAND gates.

A NAND gate is essentially an AND gate followed by a NOT gate. Therefore, to get back to the original AND logic, you must "invert the inverter" by adding another NOT gate at the end.

### Step-by-Step Implementation

- First NAND Gate: Take two inputs (A and B) and connect them to the inputs of the first NAND gate. The output of this gate is ┐ (A.B).

- Second NAND Gate (as NOT): Connect the output from the first gate into both input pins of a second NAND gate. Connecting both inputs together makes a NAND gate behave exactly like a NOT gate.

- Final Result: The second gate inverts the first output, resulting in ┐ (┐ (A.B)), which simplifies to

A.B (AND logic).

| Input A | Input B | 1st NAND Output ¬(A⋅B) | Final Output (LED) |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

- Build OR using NOR gates.

NOR gate is essentially an OR gate followed by a NOT gate, you simply need to invert the output of the first NOR gate to get back to the original OR logic.

### Step-by-Step Implementation

- First NOR Gate: Connect your two inputs (A and B) to the inputs of the first NOR gate. The output of this gate is ┐ (A+B).

- Second NOR Gate (as NOT): Connect the output from the first gate into both input pins of a second NOR gate. Just like with NAND gates, joining the inputs of a NOR gate makes it act as a NOT gate.

- Final Result: This second gate inverts the output of the first, resulting in ┐ (┐ (A + B)), which is the logic for an OR gate (A + B).

### Verification Table

| Input A | Input B | 1st NOR Output ¬(A+B) | Final Output (LED) |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 |

LAB 3: XOR AND XNOR GATES

Objective:

- Understand XOR and XNOR gates.

- Implement in Logisim.

- Verify truth tables.

Components Used:

- XOR, XNOR gates

- Input Pins

- Output LEDs

Theory:

- XOR Gate: HIGH output if number of HIGH inputs is odd. Logic: Y = AB Output is 1 if both inputs are the same.

- XNOR Gate: HIGH output if inputs are equal. Logic: Y = ¬(AB)

Commonly used in equality detectors, comparators, and parity generators.

LAB TASKS:

- Place XOR and XNOR gates in Logisim.

- Connect input pins and LEDs.

- Test all input combinations.

- Record truth tables.

Theory Questions:

- Why is XOR called exclusive OR?

- How can XNOR be used for equality detection?

Practical Questions:

- Verify XOR outputs for all inputs.

- Verify XNOR outputs for all inputs.

SMI UNIVERSITY

LAB 4: HALF ADDER AND FULL ADDER

Objective:

▪ Implement Half and Full Adders.

▪ Understand binary addition and carry.

Components Used:

- AND, OR, XOR gates

- Input Pins: A, B, Cin

- Output LEDs: Sum, Carry

Theory:

- Half Adder: Sum = AB, Carry = A·B The Sum is obtained using cascaded XOR gates.

Full Adder: Sum = A

B

Cin, Carry = (A·B)+(B·Cin)+(A·Cin)

The Carry is obtained using three AND gates and an OR gate.

A full adder can also be implemented using two half adders and an OR gate.

LAB TASKS:

- Build Half Adder in Logisim.

- Build Full Adder using  Half Adders and OR gate.

- Test all input combinations.

- Observe LED outputs.

Theory Questions:

  - Difference between Half and Full Adder.

  - Why Full Adder needs carry input?

Practical Questions:

1. Test Half Adder with all combinations.

SMI UNIVERSITY

LAB 5: HALF SUBTRACTOR AND FULL SUBTRACTOR

Objective:

Implement Half and Full Subtractors.

Understand binary subtraction and borrow.

Components Used:

- AND, OR, XOR, NOT gates

- Input Pins

- Output LEDs: Difference, Borrow

Theory:

1. Half Subtractor: Difference = AB, Borrow = ¬A·B

A Half Subtractor handles subtraction of two bits.

2. Full Subtractor: Difference = A

B

Bin, Borrow = ¬A·B + (¬A + B)·Bin

A Full Subtractor handles subtraction with a borrow from a previous stage, making it suitable for multi-bit binary subtraction.

LAB TASKS:

- Build Half Subtractor.

- Build Full Subtractor using Half Subtractors.

- Test all input combinations.

LAB 6: MULTIPLEXERS (2:1 AND 4:1)

Objective:

Understand MUX operation.

Implement 2:1 and 4:1 MUX in Logisim.

Components Used:

- AND, OR, NOT gates

- Input Pins: I0-I3

- Select Pins: S0, S1

- Output LED

Theory:

- 2:1 MUX: 2 inputs, 1 output, 1 select line.

Each term represents one input being selected.

- 4:1 MUX: 4 inputs, 1 output, 2 select lines.

Select lines and their complements ensure only one input is active at a time.

- Logic Expression: Y = I0·S1'·S0' + I1·S1'·S0 + I2·S1·S0' + I3·S1·S0 All selected terms are combined using an OR gate to produce the output.

LAB TASKS:

- Build 2:1 and 4:1 MUX in Logisim.

- Connect LED to output.

- Test all input and select combinations.

Theory Questions:

- How do select lines control output?

- Why is MUX called a data selector?

Practical Questions:

- Simulate 4:1 MUX with all inputs 0/1.

- Observe which input appears at output.

SMI UNIVERSITY

SMI UNIVERSITY

SMI UNIVERSITY

SMI UNIVERSITY

SMI UNIVERSITY

. Test Full Adder for all 8 input combinations.