Building an OR gate using NOR gates follows a similar "Universal Gate" logic to how we build an AND gate from NAND gates. Since a NOR gate is essentially an OR gate followed by a NOT gate, you simply need to invert the output of the first NOR gate to get back to the original OR logic.

### Step-by-Step Implementation

- First NOR Gate: Connect your two inputs (A and B) to the inputs of the first NOR gate. The output of this gate is ┐ (A+B).

- Second NOR Gate (as NOT): Connect the output from the first gate into both input pins of a second NOR gate. Just like with NAND gates, joining the inputs of a NOR gate makes it act as a NOT gate.

- Final Result: This second gate inverts the output of the first, resulting in ┐ (┐ (A + B)), which is the logic for an OR gate (A + B).

### Verification Table

| Input A | Input B | 1st NOR Output ¬(A+B) | Final Output (LED) |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 |